Architecture

Parts outside of pcb:

- RasberryPi4

  - Vin – 5.1V

  - 3A nominal

- STM

  - Vin - 5V usb-c

- BTS7690 Motor Drivers (3)

  - Vin 6-27V

  - Control input 3v3 – 5v

  - Peak current 43A

  - Nominal current 10-15A

- 24-12V converter

  - 30A output

- Steering actuator

  - Full load draw - max current draw – 3.2A

  - Vin – 12V

  - No load draw – 1.2A

- Brake actuator

  - Vin – 12v

  - Full load current - .24A

  - 10% DC

- Motor

  - Vin 24v

  - Max/stall current – 30-40A

  - Nominal current - 10A

- Servos

- Water Pump

- Solenoid Valve

Main Parts needed on pcb:

- Input fuse

- 12v - 5v buck

- 5v - 3.3v ldo

- Stm

- Servo motor driver

- Rails for 12v, 5v, 3v3

- 12v-6v dc-dc

NSR020A0X43Z

Part selection:

- Buck Converter

  - MCP16361T-E/NMXVAO

  - LMR51403YFDBVR

  - LMR604103SRAKR

  - BD9G401EFJ-E2

  - LMR604605SVBCRQ1

  - TI LMR60440

    - 4A input

    - Variable output

- Ldo

  - TCR2EF33,LM(CT

  - TLV70033DDCR

    - 200 mA LDO

  - AP2205-W5-7

- Input fuse

  - Littelfuse MINI-FL1

  - KRDB112.5S

  - ERDM222

- STM

  - STM32G431KB

    - datasheet

Current Draws

| Component | Nominal | max |
| --- | --- | --- |
| Steering actuator | 1.2 | 3.2 |
| Brake actuator | .55 | 1 |
| STM (sensors) | .15 | .3 |
| STM (onboard) | .1 | .2 |
| Pi4 | 2.5 | 3 |
| Servos (6) | 1 | 1.9 |
| Total | 5.5 | 10.1 |
| Motor | 10A | 30-40A |

STARTING SCHEMATIC

- MAIN COMPONENTS

  - NSR020A0X43Z

  - TLV70033DDCR

  - STM32G474CBT6TR

  - LMR51450FSQDRRRQ1

  - 1206L500/12SLPR

LMR51450FSQDRRRQ1

TLV70033DDCR

Recommends 1u caps for Cin and Cout

NSR020A0X43Z

| Pin | Name | What it does | On board |
| --- | --- | --- | --- |
| 1 | Vout | Regulated output | Connect to 6V_SERVO or 5V_SERVO rail |
| 2 | Trim+ | Output voltage set | One side of trim resistor |
| 3 | GND | Ground | Connect to main GND plane |
| 4 | PwGood | Power-good output | Optional, can go to STM input or leave unused with care |
| 5 | On/Off | Enable pin | Leave open for always-on, or control with STM/MOSFET |
| 6 | Vin | Input voltage | Connect to fused 12V rail |
| 7 | Sense+ | Remote sense positive | Usually connect to Vout / servo rail |
| 8 | Sense- | Remote sense negative | Usually connect to GND |
| 9 | Trim- | Output voltage set reference | Other side of trim resistor |
| 10 | GND | Ground/outrigger | Connect to main GND plane |

STM

NEXT STEPS

- Establish communication between Pi4 and onboard STM

  - UART between Pi and STM

    - Pi TX -> STM RX

    - Pi RX -> STM TX

    - Common GND

- Establish connection between STM and servos

  - servo receives:

    - 5V power

    - GND

    - PWM signal from STM

- Establish connection between STM and BTS7960 motor drivers

  - From STM

    - PWM

    - direction / complementary control

      - maybe enable

    - Shared logic ground

  - BTS shares gnd with power as well

- Connectors

  - Bts7690’s

    - 8 pins x2

      - 2 PWM – 3v3

      - 2 EN – 3v3

      - VCC – 5v

      - 2 Gnd

      - VCC – 12v

    - 6 pins

      - 2 PWM – 3v3

      - 2 EN – 3v3

      - VCC – 5v

      - Gnd

  - Servos

    - 3 pins x 6

      - PWM -3v3

      - VCC – 5v

      - Gnd

  - Pi4

    - 6 pins

      - 2 VCC – 5v

      - 2 Gnd

      - Tx -3v3

      - Rx – 3v3

  - Stm32

    - 4 pins

      - 2 VCC

      - 2 Gnd

  - 12v connection

    - 6 pins

      - 3 VCC

      - 3 Gnd

  - Onboard stm

    - 5 pins

      - 1 VCC – 3v3

      - 1 Gnd

      - 3 Signal

        - SWDIO

        - SWCLK

        - NRST

  - Solenoid Valve

    - 2 pins

      - 1 Gnd

      - 1 VCC – 12V

  - Water Pump

    - 2 pins

      - 1 Gnd

      - 1 VCC – 12V

| Qty | PCB connector |  |
| --- | --- | --- |
| 7 | Molex 0430450612 — Micro-Fit 3.0, 6-position, vertical, through-hole | Pi, onboard STM programming, acceleration BTS7960 logic, 3 servo connectors, offboard STM |
| 2 | Molex 39-28-1063 — Mini-Fit Jr., 6-position, vertical, through-hole | 12 V PCB input, solenoid/pump |
| 2 | Molex 39-28-1083 — Mini-Fit Jr., 8-position, vertical, through-hole | Steering BTS7960, brake BTS7960 |